# 1. Purpose

This document captures the implicit engineering decision logic behind every line of the JP narrative template. It is intended as a handoff to the ML/AI team to inform the RAG prompt, validation rules, and the AI's keep/remove decisions when generating repair steps from a Job Log input.

The original template uses color coding to distinguish near-mandatory standard text (blue) from conditional content (green). This markup makes the conditional logic explicit — for each conditional line, it defines (a) when the line should be kept, (b) what input data drives the decision, and (c) where that data comes from.

# 2. The five decision patterns

Every conditional element in the template falls into one of seven patterns. Patterns A–D were defined in v1.0; E–G were added in v1.1 based on EWO validation.

▸ Pattern A — Pure include/exclude

The line is either kept verbatim or removed entirely.

Example: Line 42 — '[Only when dismantling piping with existing spring support…] A spring stopping pin must be installed…'

▸ Pattern B — Include + substitute

The line is always included for in-scope work, but contains red placeholders that must be filled.

Example: Line 23 — '…isometric drawing 00-YYYY-L-ZZZZ.' Substitute with the actual drawing ID.

▸ Pattern C — Include + reference pruning

The line is always included, but contains an OR-list of spec references or workflow branches of which only some apply.

Example: Line 30 — '…in compliance with TES PIM-SU-2505/W-ST-2011/2016/2026/2028.' Prune to only the welding spec matching material and service.

▸ Pattern D — Engineer instruction (meta)

Purple-text directives in square brackets are guidance to the JP author, not part of the final output. AI must read them as gating conditions and strip them from output.

Example: Line 20 — 'Use when insufficient data available for DTEC to verify Pressure Vessel…'

▸ Pattern E — Engineer-added custom content

Beyond keep/remove of template lines, engineers add project-specific risk controls, named contacts, site-specific NDE matrices. AI should be permitted to add steps when the scope warrants.

Example: EWO 2013037 added 'Install temporary barriers around construction site, at least 1m from structures that are in repair' — site-specific hazard control.

# 3. Canonical input variables

Before the AI can decide keep/remove on any conditional line, it must have these 31 inputs available. Some come from Salesforce Job Log directly, some via RAG lookup on TCO specs.

| Variable | Source | Examples / values | Note |
| --- | --- | --- | --- |
| line_class | P&ID line number + lookup PIM-SU-5112-TCO | 150H21, 300K21 | Primary RAG key. Resolves material, NDE class, PWHT, coating via single lookup. |
| material_family | Lookup MATERIAL column of PIM-SU-5112-TCO | LTCS / SS / LTCS GALV / HDPE | CS – Carbon Steel LTCS – Low Temperature Carbon Steel SS- Stainless Steel |
| material | Derived from line class within L-ST-2056 | A-105, A-333 Gr.6, A-312 TP316/L | ASME steel grades |
| service | Mark-up P&ID – line number – Piping Line Class – Fluid Code, Job Log problem statement, PIM-SU-5112 | PHC – PROCESS HYDROCARBONS DHC - DRAIN, HYDROCARBONS | — |
| design_T_max | P&ID or explicitly provided | °C | Essential for Hyrdotest pressure correction factor |
| design_T_min | P&ID or explicitly provided | °C | Drives LTCS/cryogenic choice |
| design_P | P&ID or explicitly provided | barg | Defines hydrotest pressure as per B31.3 |
| insulation_type | - Mark-up P&ID – line number – Piping Line Class- Insulation Designation. - PIM-SU-5112 – table 10.0 PIPING SYSTEMS INSULATION SELECTION TABLE | The most common: HC - Heat Conservation HCDW’X’- Heat Conservation Double Layer & Electric Tracing (‘X’ denotes tracing temperature) NI - Not Insulated | Drives insulation removal/reinstate + spec choice (IRM-SU-1381 vs IRM-SU-2634) Note: IRM-SU-2634 shall be ignored for MVP phase. |
| heat_tracing | Mark-up P&ID – line number – Piping Line Class- Insulation Designation (Insulation code). or symbol on PID | None / Electric / Steam / Water / Contro-Trace | Drives I&E coordination steps + chloride-cracking risk on SS Note: Contro-Trace shall be ignored for MVP phase. |
| scope_type | Job Log (Job Description), Inspection Report, Anomaly Report or explicitly provided. | Piping section replacement / TLR removal with pipe section replacement/ Flange replacement | Defines how defect will be addressed, meaning repaired. |
| pwht_required | PIM-SU-2505 → material + thickness + WPS Lookup PWHT column of PIM-SU-5112-TCO L-ST-2056 → Line class → PWHT FOR WT > 19 MM TCO Welding Procedures Selection Spreadsheet for Pipe Classes specified in L-ST-2056 Rev 4 → Line class → Y or N | Yes / No | Triggers PWHT steps and ordering of NDE after PWHT |
| ht_required | TCO NDE requirements per class _25.03.20 R03 (document) → Line class → NDE Requirements → HT | % = 0 or 100 and iif (%=100) Max value = 200 / 235 HB | Defines piping Hardnes Test max value after PWHT Note: Shall be performed only after PHWT. |
| nde_class | TCO NDE requirements per class _25.03.20 R03 (document) → Line class → NDE Requirements | RT / UT / MT / PT / VT methods | RT (Radiographic Testing) UT (Ultrasonic Testing) MT (Magnetic Particle Testing) PT (Dye Penetrant Testing) VT (Visual Testing) |
| bevel_inspection_methods | material + service → PIM-SU-2505 / W-ST-2025 | ['VT-100%', 'MT-100%'] or ['PT-100%', 'MT-100%'] or ['MT-100%'] only |  |
| coating_system | insulation_type (variable) + material_family (variable) → COM-SU-5191 (Table 1 and 3) | The most common: System 3.2 (LTCS + NI) System 12.1 (LTCS + HCW) | — |
| existing_spring_support_reuse | ISO or Job Log (description), | Yes / No | Most of the time existing spring supports are reused. |
| alloy_pmi_required | TCO NDE requirements per class _25.03.20 R03 (document) → Line class → NDE Requirements → PMI Alternatively (rule of thumb): material_family (variable) ≠ CS/LTCS → PMI 100% | Yes (100%) / No (Blank) | Should be performed prior fabrication |
| chloride_risk | (material_family (variable) = SS) + heat_tracing (variable) | Yes / No | — |
| hydrotest_method | All B31.3 line classes are subjected to hydrotest. Exception: Cat. D fluid only = service test in leu of hydrotest. If it meets all of these rules: service: Safe (non-flammable, non-toxic, won't burn skin). design_P: ≤ 10.3 bar design_T_max: Between -29°C to 186 °C. NO PNEUMATIC! | full_hydrostatic / service_test_only / pneumatic / none |  |
| placeholders_TP | explicitly provided tie_in_ids in prompt scema or can be derived from scope_type = Piping section replacement | TP25-0424/001, TP EWO 2013037/002 | Substitutes red placeholder TPxx-yyy/000 TP (tie-in point) |
| placeholders_dwg | explicitly provided in ISO or sketch L-SK-00X is default naming | L-SK-001, L-SK-002, 020-0900-LLL-ISO-20028-01 | — |
| spool_prefab | scope_type = Piping section replacement Scope + ISO | Yes / No | — |

# 5. Line-by-line annotation

Each line is annotated with: paragraph ID (from template), color tag, decision pattern, the keep/remove rule, inputs required, and RAG sources. Lines marked 'v1.1' in their notes received corrections from EWO validation.

## 5.1. HEADER

Line 7  ·  Color: BLK  ·  Pattern Structural

Text: "PRESENT SITUATION:"

Keep when: Always

Remove when: Never

## 5.2. PRESENT SITUATION

Line 8  ·  Color: BLU2/RED  ·  Pattern B

Text: "Problem Statement derived from the EWR."

Keep when: Always — replace with actual problem statement from Job Log

Remove when: Never

Inputs needed: problem_statement

RAG sources: Salesforce Job Log → Job Description

## 5.3. HEADER

Line 9  ·  Color: BLK  ·  Pattern Structural

Text: "PROPOSED SOLUTION:"

Keep when: Always

Remove when: Never

## 5.4. PROPOSED SOLUTION

Line 10  ·  Color: RED  ·  Pattern B

Text: "Proposed Solution presented in this Job Pack."

Keep when: Always — replace with one-paragraph solution summary

Remove when: Never

Inputs needed: scope_type, line_class, placeholders_TP

RAG sources: Explicitly provided (MVP) or Salesforce Job Log → Job Description (Product)

## 5.5. HEADER

Line 11  ·  Color: BLK  ·  Pattern Structural

Text: "SCOPE OF WORK"

Keep when: Always

Remove when: Never

## 5.6. SCOPE OF WORK

Line 12  ·  Color: BLU  ·  Pattern Always-include

Text: "BUSINESS PARTNER (BP) shall strictly follow all relevant TCO Safety Instructions."

Keep when: Always

Remove when: Never

Line 13  ·  Color: BLU  ·  Pattern Always-include

Text: "BUSINESS PARTNER (BP) shall follow all Quality Management (QM) requirements designated in the Quality Control package."

Keep when: Always

Remove when: Never

## 5.7. PRELIMINARY SITE WORK

Line 14  ·  Color: GRN/Bold  ·  Pattern A

Text: "PRELIMINARY SITE WORK (PSW):

Keep when: When any PSW activity applies (almost always)

Remove when: Never

Inputs needed: scope_type

Line 15  ·  Color: GRN/RED  ·  Pattern B + A

Text: "Prior to start of mechanical activities, perform wall thickness testing at TP-EWR number /000 and TP-EWR number /999 to confirm that wall thickness is adequate for welding. Results shall be reviewed and approved by the Fixed Equipment Reliability (FER) Unit Inspector.""

Keep when: placeholders_TP (tie_in_ids) is provided.

Remove when: Only if no tie-ins

Inputs needed: placeholders_TP, scope_type

Note: Ask for tie-in numbers if wasn’t explicitly provided. If not provided, given one should be displayed in RED font.

Line 16  ·  Color: GRN  ·  Pattern A-paired

Text: "NOTE: Perform the wall thickness test by marking out and recording a matrix as directed by the TCO FER Unit Inspector."

Keep when: Whenever line 15 is kept

Remove when: Whenever line 15 is removed

Line 17  ·  Color: GRN  ·  Pattern A

Text: "Prior to commencing any fabrication works verify dimensions, pipe routing and field weld locations on site."

Keep when: When fabrication in scope (almost always)

Remove when: Never

Inputs needed: scope_type

Line 18a  ·  Color: GRN/PUR  ·  Pattern A + C  ·  (OR-split of Line 18)

Text: NOTE: When allocating FW & FFW locations for machinery nozzle connections, consider the possibility of final piping adjustment after nozzle alignment completion.

Keep when: scope_type = machinery connection present, connected to machinery nozzle. Never print.

Remove when: otherwise (almost always for LCW)

Inputs needed: scope_type

Note: One of three OR-triggers for the same note (see 18b/18c). Ask back if multiple triggers are true.

Line 18b  ·  Color: GRN/PUR  ·  Pattern A + C  ·  (OR-split of Line 18)

Text: NOTE: When allocating FW & FFW locations for fixed equipment nozzle connections, consider the possibility of final piping adjustment after nozzle alignment completion.

Keep when: scope_type = connected to fixed equipment nozzle, connected to vessel nozzle

Remove when: otherwise (almost always for LCW)

Inputs needed: scope_type

Note: OR-sibling of 18a/18c; Ask back if multiple triggers are true.

Line 18c  ·  Color: GRN/PUR  ·  Pattern A + C  ·  (OR-split of Line 18)

Text: NOTE: When allocating FW & FFW locations for PSV inlet/outlet connections, consider the possibility of final piping adjustment after nozzle alignment completion.

Keep when: scope_type = PSV connection, connected to PSV inlet, connected to PSV outlet.

Remove when: otherwise (almost always for LCW)

Inputs needed: scope_type

Note: OR-sibling of 18a/18b; Ask back if multiple triggers are true.

Line 20  ·  Color: GRN/PUR  ·  Pattern D

Text: "[Use when insufficient data available for DTEC to verify Pressure Vessel internal dimensions or configuration prior to AFC issue]"

Keep when: Never print — gates line 21

Remove when: Always strip

Inputs needed: scope_type

Line 21  ·  Color: GRN/RED  ·  Pattern A

Text: "NOTE: Invite Responsible Engineer to verify dimensions / internal configuration / XXX on site, once vessel is isolated and full access is provided."

Keep when: Never print

Remove when: Always strip

Inputs needed: scope_type

Line 22  ·  Color: GRN  ·  Pattern A

Text: "Prior to excavation works, for installation of pipe support foundation(s) in unpaved areas, perform surveying work for the presence of underground utilities. Arrange this with the TCO Lead Field Surveyor, phone 4931." Prior to excavation works, for installation of pipe support foundation(s) in unpaved areas, perform surveying work for the presence of underground utilities. Arrange this with the TCO Lead Field Surveyor, phone 4931."

Keep when: scope_type = new piping route (almost never for low complexity scope)

Remove when: Always strip

Inputs needed: scope_type

Line 23  ·  Color: GRN/RED/BLU  ·  Pattern A

Text: "Perform excavation works and install concrete foundation of new unpaved area pipe support(s) as per isometric drawing(s) 00-YYYY-L-ZZZZ and the attached drawing from Tengiz Engineering Specification (TES) L-ST-6075."

Keep when: Line 22 is kept

Remove when: Otherwise

Line 24  ·  Color: GRN/PUR  ·  Pattern D

Text: Consider the weight of the new equipment indicated on drawing XXXX during developing execution steps and lifting plan."

Keep when: scope_type = pressure vessel replacement, valve replacement (size >= 4”)

Remove when: otherwise (almost always for LCW)

Inputs needed: scope_type

Line 25  ·  Color: GRN  ·  Pattern A

Text: " NOTE: DTEC completed required engineering studies of existing/new piping, foundation & supports and confirmed new equipment weight is within allowable loads.

Keep when: Never

Remove when: Otherwise

## 5.8. SHOP WORK

Line 26  ·  Color: BLK  ·  Pattern Structural

Text: "SHOP WORK:"

Keep when: Always

Remove when: scope_type = valve replacement (only)

Inputs needed: scope_type, spool_prefab

Line 27  ·  Color: BLU  ·  Pattern Always-include

Text: "Withdraw materials required for this Job Pack from TCO Warehouse according to Material Request(s)."

Keep when: Always

Remove when: Never

Line 28  ·  Color: GRN  ·  Pattern A

Text: "Perform Positive Material Identification (PMI) as per TCO RE QM SWP-20 Procedure on all corrosion resistant alloy (CRA) materials and weld consumables."

Keep when: alloy_pmi_required = true (material is SS/alloy/duplex) alternatively line class base family contains P

Remove when: material family = LTCS / LTCS GALV / HDPE

Inputs needed: material_family, alloy_pmi_required

RAG sources: TCO NDE requirements per class _25.03.20 R03, PIM-SU-5112

Line 29a  ·  Color: BLU/GRN/RED  ·  Pattern B + C

Text: "Prefabricate pipe spools as per isometric drawing(s) 00-YYYY-L-ZZZZ. Use TCO-approved Welding Procedure Specification(s) (WPSs) indicated on isometric drawing(s).

Keep when: spool_prefab =  yes. (almost always)

Remove when: Otherwise

Inputs needed: spool_prefab, placeholders_dwg

Line 29b  ·  Color: BLU/GRN/RED  ·  Pattern B + C

Text: "Prefabricate pipe spools and pipe supports, as per isometric drawing(s) 00-YYYY-L-ZZZZ. Use TCO-approved Welding Procedure Specification(s) (WPSs) indicated on isometric drawing(s).”

Keep when: ISO or Sketch ∈ PIPE SUPPORT ≠ 0. (Note: Meaning pipe supports present in construct drawing. Potentially, it is a result of phase 2 when drawing is parsed and tabular data retrieved.)

Remove when: Otherwise

Inputs needed: spool_prefab, placeholders_dwg + PIPE SUPPORT ≠ 0

Line 29a/b + c ·  Color: BLU/GRN/RED  ·  Pattern B + C

Text: (29a or 29b text) + “BP shall comply with TCO Guideline 16-0015-MI for socket-welded joints.”

Keep when: ISO or Sketch ∈ SHOP MATERIAL ∈  Sockolet. (Note: Meaning MTO construct drawing indicates Sockolet. Potentially, it is a result of phase 2 when drawing is parsed and tabular data retrieved.

Remove when: Otherwise

Inputs needed: spool_prefab, placeholders_dwg + SHOP MATERIAL ∈  Sockolet

Line 30a  ·  Color: GRN/RED  ·  Pattern A (OR-split of Line 30)

Text: "Perform PWHT as specified by WPS and per TES PIM-SU-2505"

Keep when: pwht_required=true

Remove when: Otherwise

Inputs needed: pwht_required

RAG sources: PIM-SU-2505, TCO Welding Procedures Selection Spreadsheet for Pipe Classes specified in L-ST-2056 Rev 4

Line 30b  ·  Color: GRN/RED  ·  Pattern A (OR-split of Line 30)

Text: "Perform PWHT as specified by WPS and per TES W-ST-2011."

Keep when: pwht_required = Yes and iif scope_type = pipeline

Remove when: Otherwise (almost always for LCW)

Inputs needed: pwht_required

RAG sources: W-ST-2011, TCO Welding Procedures Selection Spreadsheet for Pipe Classes specified in L-ST-2056 Rev 4

Line 30c  ·  Color: GRN/RED  ·  Pattern A (OR-split of Line 30)

Text: "Perform PWHT as specified by WPS and per TES W-ST-2016."

Keep when: scope_type = gas injection

Remove when: Always

Inputs needed: scope_type, pwht_required

RAG sources: W-ST-2016, TCO Welding Procedures Selection Spreadsheet for Pipe Classes specified in L-ST-2056 Rev 4

Line 30d  ·  Color: GRN/RED  ·  Pattern A (OR-split of Line 30)

Text: "Perform PWHT as specified by WPS and per TES W-ST-2026."

Keep when: scope_type = wellhead

Remove when: Always

Inputs needed: scope_type, pwht_required

RAG sources: W-ST-2026, TCO Welding Procedures Selection Spreadsheet for Pipe Classes specified in L-ST-2056 Rev 4

Line 30e  ·  Color: GRN/RED  ·  Pattern A (OR-split of Line 30)

Text: "Perform PWHT as specified by WPS and per TES W-ST-2028."

Keep when: scope_type = heavy wall piping

Remove when: Always

Inputs needed: scope_type, pwht_required

RAG sources: W-ST-2028, TCO Welding Procedures Selection Spreadsheet for Pipe Classes specified in L-ST-2056 Rev 4

Line 31a  ·  Color: BLU/GRN/RED  ·  Pattern B (OR-split of Line 31)

Text: “Perform non-destructive examination (NDE) as specified on isometric drawing(s) 00-YYYY-L-ZZZZ”

Keep when: Line 29a/b/ac/bc is kept but Line 30a is dropped (alternatively: and pwht_required = No.)

(Note: Values also can be taken in phase 2 when drawing is parsed and tabular data is retrieved.)

Remove when: Line 29a/b/ac/bc is dropped.

Inputs needed: spool_prefab, placeholders_dwg

Line 31b  ·  Color: BLU/GRN/RED  ·  Pattern B

Text: “Perform non-destructive examination (NDE) and hardness testing of shop welds, as specified on isometric drawing(s) 00-YYYY-L-ZZZZ”

Keep when: Line 29a/b/ac/bc and Line 30a are kept (alternatively: and pwht_required = Yes)

(Note: Values also can be taken in phase 2 when drawing is parsed and tabular data is retrieved.)

Remove when: Line 30a is dropped.

Inputs needed: spool_prefab, placeholders_dwg, nde_class

Line 32  ·  Color: GRN  ·  Pattern A

Text: "NOTE: Perform NDE and hardness testing only after PWHT!"

Keep when: Line 31b is kept

Remove when: Otherwise

Line 33  ·  Color: GRN  ·  Pattern

Text: "NOTE: BP must check with MaxTrax admin on NDE requirements prior to commence welding works.”

Keep when: Line 29a/b/ac/bc is kept

Remove when: Otherwise

Line 34a  ·  Color: BLU/PUR/GRN/RED  ·  Pattern A + B (OR-split of Line 34)

Text: "Perform shop hydrostatic testing of flanged spools as specified on isometric drawing(s) 00-YYYY-L-ZZZZ in accordance with TES PIM-SU-3541-TCO."

Keep when: hydrotest_method ≠ service test and spool_prefab=true. (Note: Values also can be taken in phase 2 when drawing is parsed and tabular data is retrieved.)

Remove when: hydrotest_method = service test. Following should be true:

service: Safe (non-flammable, non-toxic, won't burn skin).

design_P: ≤ 10.3 bar

design_T_max: Between -29°C to 186 °C.

Inputs needed: spool_prefab, placeholders_dwg, hydrotest_method

RAG sources: PIM-SU-3541-TCO

Line 34b  ·  Color: BLU/PUR/GRN/RED  ·  Pattern A + B (OR-split of Line 34)

Text: "Perform shop hydrostatic testing of ASME B31.4 / B31.8 pipeline components in accordance with TCO Procedure X-000-L-PRO-0001 as per isometric drawing(s) 00-YYYY-L-ZZZZ – 4-hour duration.

Keep when: Line 29a/b/ac/bc and Line 30b is kept.

Remove when: Line 30b is dropped

Inputs needed: spool_prefab, placeholders_dwg, hydrotest_method

RAG sources: PIM-SU-3541-TCO

Line 35  ·  Color: GRN  ·  Pattern A

Text: " NOTE: Test water used for 300-series stainless steel piping shall be water with chloride content of less than 50 ppm. Perform Water Sampling and Analysis per TES PIM-SU-3541-TCO Section 5."

Keep when: Line 34a is kept and material_family = SS. (Note: Values also can be taken in phase 2 when drawing is parsed and tabular data is retrieved.)

Remove when: material_family = LTCS / LTCS Galv / HDPE

Inputs needed: material_family

RAG sources: PIM-SU-3541-TCO

Line 36  ·  Color: GRN/BLU2/RED  ·  Pattern A + B

Text: "Abrasive blast and coat new piping spools and supports, as specified on isometric drawing 00-YYYY-L-ZZZZ in accordance with TES COM-SU-5191-TCO."

Keep when: material_family = LTCS (only)

Remove when: Line 35 is kept or material_family = LTCS Galv / SS / HPDE

Inputs needed: coating_system, material_family, placeholders_dwg

RAG sources: COM-SU-5191-TCO

Line 37  ·  Color: GRN  ·  Pattern A

Text: "NOTE: Mask-off field-weld joints and flange faces prior to coating."

Keep when: Whenever line 36 is kept.

Remove when: Whenever line 36 is removed

## 5.9. SITE WORK

Line 38  ·  Color: BLK  ·  Pattern Structural

Text: "SITE WORK: (section header)"

Keep when: Always for in-kind

Remove when: Never

Line 39  ·  Color: BLU  ·  Pattern Always-include

Text: "Perform relevant job safety assessments (PPHA & JSA) and obtain required Permits-to-Work (PTWs).”

Keep when: Always

Remove when: Never

RAG sources: SI-105

Line 40a  ·  Color: GRN/BLU  ·  Pattern A

Text: "Demarcate work area as necessary and erect scaffolding for access."

Keep when: Always

Remove when: Never

Line 40b  ·  Color: BLU/PUR/GRN/RED  ·  Pattern A + B (OR-split of Line 34)

Text: "NOTE: TCO Operations shall isolate, depressurize, drain and ready for Hot Work, Equipment and Piping associated with this Project, if and as applicable, and shall hand it over to BUSINESS PARTNER (BP); fully accessible, safe and Locked-Out-Tagged-Out (LOTO) as scheduled and agreed between TCO Operations and BP.”

Keep when: Line 40a is kept.

Remove when: Line 40a is dropped

Line 41  ·  Color: GRN/BLU2  ·  Pattern A + D

Text: "A spring stopping pin must be installed before pipe dismantling. Note that the pin must be removed on the same condition as it was installed. If pipe was full at installation of pin, new pipe must be filled prior to pin removal. Spring level / indicator must be marked on hot and cold conditions. "

Keep when: existing_spring_support_reuse = true

Remove when: Otherwise

Inputs needed: existing_spring_support_reuse

Line 42  ·  Color: GRN  ·  Pattern A

Text: "Remove all cladding and insulation to the extent required."

Keep when: insulation_type != NI or prompt contains Insulation = Yes

Remove when: insulation_type = NI or prompt contains Insulation = No. Meaning explicitly provided.

Inputs needed: insulation_type, prompt (Insulation)

RAG sources: IRM-SU-1381

Note: Ask for insulation details if no details are explicitly provided.

Line 43 ·  Color: GRN/BLU2  ·  Pattern A + D

Text: "Check list MCL005A-1 (Spring Support Verification for Maintenance) shall be completed and signed by responsible parties.”

Keep when: Line 40a is kept.

Remove when: Otherwise

Inputs needed: existing_spring_support_reuse

Line 44a  ·  Color: GRN/BLU2  ·  Pattern A + D

Text: "Invite Machinery Maintenance team to install dial gauges prior to existing piping dismantle to record “Zero” position of the compressor/pump."

Keep when: scope_type = machinery connection present, connected to machinery nozzle. Never print.

Remove when: otherwise (almost always for LCW)

Inputs needed: scope_type

Line 44b  ·  Color: GRN  ·  Pattern A

Text: "NOTE: Gauges stay on until all work complete. Last step is record of gauges and removal."

Keep when: Line 44a is kept.

Remove when: Otherwise

Inputs needed: scope_type

Line 45a  ·  Color: GRN/PUR/RED  ·  Pattern A + C

Text: "Arrange with Tengiz Turnaround Team (TTT) I&E Coordinator [OR]  Zone Maintenance to isolate, LOTO and remove electric trace heating and instruments from associated piping per I&E Job Pack YY-NNNN.1 "

Keep when: prompt contains Heat tracing = electric

Remove when: Heat tracing = None

Inputs needed: heat_tracing

Note: I&E Job Pack YY-NNNN.1 exact number should be asked. If not provided, given one should be displayed in RED font.

Line 45b

Line 45b  ·  Color: GRN/PUR/RED  ·  Pattern A + C

Text: "Carefully remove for reinstatement (or Demolish) hot water / steam tracing where necessary."

Keep when: prompt contains Heat tracing = water, steam or Contro-Trace

Remove when: Heat tracing = None

Inputs needed: heat_tracing

Note: OR-sibling of 45a; Ask back if multiple triggers are true. Ask for heat tracing type if no details are explicitly provided keep both types in RED font.

Line 45c  ·  Color: GRN  ·  Pattern A

Text: " NOTE: BP to match mark, call a unit operator, show the mark and record location prior to cutting and removal to aid in reinstatement of heat tracing.”

Keep when: Line 45a is kept.

Remove when: Otherwise

Inputs needed: heat_tracing

Line 46  ·  Color: GRN  ·  Pattern B

Text: "Apply and sign special tape, which determines the place of cutting the pipe, according to SP-26 at TP-XXX. Follow all the steps of SP-26."

Keep when: Line 15 is kept.

Remove when: Otherwise

Inputs needed: placeholders_TP

Note: TP-XXX should propagate from line 15 ask back result.

Line 47a  ·  Color: BLU/PUR/GRN/BLU2  ·  Pattern B

Text: "Cold Cut, unbolt and remove existing piping and pipe supports shown on “Destruct Detail” of isometric drawing(s) 00-YYYY-L-ZZZZ.”

Keep when: Line 46 is kept

Remove when: Otherwise

Inputs needed: placeholders_dwg

RAG sources: Isometric drawing (Desctruct)

Note: 00-YYYY-L-ZZZZ exact number should be asked. If not provided, template placeholder should be displayed in RED font.

Line 47b  ·  Color: BLU/PUR/GRN/BLU2  ·  Pattern B

Text: "Make good any remaining foundations and/or metal structures and perform coating repairs per TES COM-SU-5191-TCO.”

Keep when: Line 47a is kept

Remove when: Otherwise

Inputs needed: placeholders_dwg

RAG sources: Isometric drawing (Desctruct)

Line 47c  ·  Color: BLU/PUR/GRN/BLU2  ·  Pattern B

Text: "Remove the redundant plinths and/or metal structures, supports.

Keep when: scope_type = new piping route (almost never for low complexity scope)

Remove when: Always strip

Inputs needed: scope_type

Note: OR-sibling of 47b.

Line 48  ·  Color: GRN/BLU2/RED  ·  Pattern A + B

Text: "Perform hydrogen bake-out in the weld zone of each tie-in points as shown on isometric drawing(s) 00-YYYY-L-ZZZZ following requirements in TES PIM-SU-2505.

Keep when:  line_class ∈ letter (K,C80 and A80) or service (fluid code) ∈ {GS, WSR, CDE, CDF, FX, FXE, FXF, GRG, GSA or explicit wet-sour statement in EWR.

Remove when: Otherwise (non-sour service)

Inputs needed: line_class, service (fluid code)

RAG sources: TES PIM-SU-2505

Note 1: If KEEP WHEN is satisfied, the need for bake-out should be asked back as:

- Have you contacted with MI engineer and confirmed the need for Hydrogen Bakeout?

If answered yes,  insert the text as regular. If answered no (not yet), insert text in RED font.

Note 2: 00-YYYY-L-ZZZ number should propagate from Line 47a.

Line 49  ·  Color: GRN/RED  ·  Pattern B

Text: "Bevel cut ends of existing piping at the tie-in point(s) TPxx-yyy/000 for field welding as shown on isometric drawing(s) 00-YYYY-L-ZZZZ."

Keep when: Line 47a is kept

Remove when: Bolted-flange only

Inputs needed: placeholders_TP, placeholders_dwg

Note: TP-XXX should propagate from line 15 ask back result. 00-YYYY-L-ZZZ number should propagate from Line 47a.

Line 50a  ·  Color: GRN  ·  Pattern A

Text: " Inspect the prepared weld bevel ends 100% VT and 100 % MT/PT for defects”

Keep when: Line 49 is kept

Remove when: Otherwise

Inputs needed: bevel_inspection_methods

Line 50b  ·  Color: GRN  ·  Pattern A

Text: " NOTE: Perform inspection of weld bevels and nipo-flange landings PRIOR to welding.."

Keep when: Line 50a is kept

Remove when: Otherwise

Line 51  ·  Color: GRN  ·  Pattern A

Text: " Perform PMI at field joints as per TCO RE QM SWP 20 Procedure on alloy materials listed in SWP-20 Appendix 20-1.”

Keep when: alloy_pmi_required = yes or material_family = SS

Remove when: alloy_pmi_required = yes or material_family = LTCS and HDPE

Inputs needed: material_family, alloy_pmi_required

Line 52a  ·  Color: BLU/GRN/RED  ·  Pattern B + C

Text: "Install piping as specified on isometric drawing 00-YYYY-L-ZZZZ. For field welds use TCO-approved WPSs indicated on isometric drawing(s)."

Keep when: Line 47a is kept

Remove when: Never

Inputs needed: placeholders_dwg

RAG sources: Isometric drawing

Note: 00-YYYY-L-ZZZ number should propagate from Line 47a.

Note: ISO or Sketch ∈ PIPE SUPPORT ≠ 0. (Note: Meaning pipe supports present in construct drawing. Potentially, it is a result of phase 2 when drawing is parsed and tabular data retrieved.)

Line 52b  ·  Color: BLU/GRN/RED  ·  Pattern B + C

Text: "BP shall comply with TCO Guideline 16-0015-MI for socket-welded joints.”

Keep when: ISO or Sketch ∈ FIELD MATERIAL ∈  Sockolet. (Note: Meaning MTO construct drawing indicates Sockolet. Potentially, it is a result of phase 2 when drawing is parsed and tabular data retrieved

Remove when: Otherwise

Inputs needed: placeholders_dwg

RAG sources: Isometric drawing

Note: 00-YYYY-L-ZZZ number should propagate from Line 47a.

Line 52b  ·  Color: BLU/GRN/RED  ·  Pattern B + C

Text: "BP shall comply with additional weld controls denoted in Closure Weld Package(s) CW-00-0YYY/00X.”

Keep when: scope_type ∈ Closure weld (Usually not LCW)

Remove when: Otherwise

Inputs needed: scope_type

RAG sources: Isometric drawing

Note: CW-00-0YYY/00X number should be provided explicitly.

Line 53  ·  Color: GRN  ·  Pattern A

Text: " NOTE: Do NOT drill the header or cut out nipo-flange coupons prior to hydro-testing branch connections.”

Keep when: ISO or Sketch ∈ FIELD MATERIAL ∈  Nipoflange. (Note: Meaning MTO construct drawing indicates Sockolet. Potentially, it is a result of phase 2 when drawing is parsed and tabular data retrieved.

Remove when: Otherwise

RAG sources: Isometric drawing

Line 54  ·  Color: GRN/PUR  ·  Pattern A + D

Text: "NOTE: Verify the piping alignment with respect to machinery suction/discharge nozzle before the field welding. Flange alignment shall be parallel to pump nozzle with ability to insert/remove bolting without any binding.  Flange-face gaps shall only accommodate Gasket including Spec Blind as applicable. API 686 shall be followed for alignment requirements.”

Keep when: scope_type = machinery connection present, connected to machinery nozzle. Never print.

Remove when: Otherwise (almost always for LCW)

Inputs needed: scope_type

Line 55  ·  Color: BLK/GRN/RED  ·  Pattern A + B

Text: "Install civil structural support as per isometric drawing 00-YYYY-L-ZZZZ and attached Standard Support drawing.."

Keep when: ISO or Sketch ∈ PIPE SUPPORT ≠ 0. (Note: Meaning pipe supports present in construct drawing. Potentially, it is a result of phase 2 when drawing is parsed and tabular data retrieved. Additionally, function of pulling standard support drawings is tied to this step.)

Remove when: Otherwise

Inputs needed: spool_prefab, placeholders_dwg + PIPE SUPPORT ≠ 0

RAG sources: PIM-DU-5153-TCO, Standard Support drawings

Note: 00-YYYY-L-ZZZ number should propagate from Line 47a.

Line 56  ·  Color: GRN/RED  ·  Pattern A + C

Text: "Perform PWHT of field joints as specified by WPS and in compliance with TES PIM-SU-2505."

Keep when: pwht_required = true or ISO or Sketch ∈ PWHT ≠ 0. (Note:, it is a result of phase 2 when drawing is parsed and tabular data retrieved.)

Remove when: Otherwise

Inputs needed: pwht_required + PWHT ≠ 0

RAG sources: PIM-SU-2505, TCO Welding Procedures Selection Spreadsheet for Pipe Classes specified in L-ST-2056 Rev 4

Note: Other specs (2011-2016) shall be ingored for LCW. Subject for implementation during scaling.

Line 57a  ·  Color: BLU/GRN/RED  ·  Pattern B (OR-split of Line 57)

Text: "Perform NDE of field welds, as specified on isometric drawing(s) 00-YYYY-L-ZZZZ.”

Keep when: Line 52a is kept but Line 56 is dropped (alternatively: pwht_required = No.) (Note: Values also can be taken in phase 2 when drawing is parsed and tabular data is retrieved.)

Remove when: Line 52a is dropped.

Inputs needed: placeholders_dwg

Line 57b  ·  Color: BLU/GRN/RED  ·  Pattern B (OR-split of Line 57)

Text: "Perform NDE and hardness testing of field welds, as specified on isometric drawing(s) 00-YYYY-L-ZZZZ.”

Keep when: Line 52a and Line 56 are kept (alternatively: pwht_required = Yes.) (Note: Values also can be taken in phase 2 when drawing is parsed and tabular data is retrieved.)

Remove when: Line 56 is dropped.

Inputs needed: placeholders_dwg

Line 57c  ·  Color: BLU/GRN/RED  ·  Pattern B

Text: "BP shall comply with specified NDE requirements denoted in closure weld package(s) CW-00-0YYY/00X”

Keep when: scope_type ∈ Closure weld (Usually not LCW)

Remove when: otherwise

Inputs needed: scope_type

RAG sources: Isometric drawing

Note: CW-00-0YYY/00X number should be provided explicitly.

Line 57c  ·  Color: BLU/GRN/RED  ·  Pattern B

Text: "NOTE: Perform NDE and Hardness Testing only after PWHT!”

Keep when: Line 57b is kept

Remove when: otherwise

Inputs needed: scope_type

RAG sources: Isometric drawing

Line 57d  ·  Color: BLU/GRN/RED  ·  Pattern B

Text: "NOTE: BP must check with MaxTrax admin on NDE requirements prior to commence welding works.”

Keep when: Line 57a or 57b is kept.

Remove when: otherwise

Inputs needed: scope_type

RAG sources: Isometric drawing

Line 58a  ·  Color: BLU/PUR/GRN/RED  ·  Pattern A + B  (OR-split of Line 58)

Text: "Perform field hydrostatic testing of new piping as specified on isometric drawing(s) 00-YYYY-L-ZZZZ in accordance with TES PIM-SU-3541-TCO”

Keep when: Line 52a is kept. (Note: Values also can be taken in phase 2 when drawing is parsed and tabular data is retrieved.)

Remove when: Otherwise

Inputs needed: hydrotest_method, placeholders_dwg]

RAG sources: PIM-SU-3541-TCO

Note: 00-YYYY-L-ZZZ number should propagate from Line 47a.

Line 58b  ·  Color: BLU/PUR/GRN/RED  ·  Pattern A + B (OR-split of Line 58)

Text: "Perform shop hydrostatic testing of ASME B31.4 / B31.8 pipeline components in accordance with TCO Procedure X-000-L-PRO-0001 as per isometric drawing(s) 00-YYYY-L-ZZZZ – 4-hour duration.

Keep when: Line 52a and iif scope_type = pipeline.

Remove when: Otherwise

Inputs needed: placeholders_dwg, hydrotest_method, scope_type

RAG sources: PIM-SU-3541-TCO

Note: 00-YYYY-L-ZZZ number should propagate from Line 47a.

Line 58c  ·  Color: BLU/PUR/GRN/RED  ·  Pattern A + B (OR-split of Line 58)

Text: "NOTE: Test water used for 300-series stainless steel piping shall be water with chloride content less than 50 ppm.  Perform Water Sampling and Analysis per TES PIM-SU-3541-TCO Section 5.”

Keep when: Line 58a is kept and material_family = SS. (Note: Values also can be taken in phase 2 when drawing is parsed and tabular data is retrieved.)

Remove when: material_family = LTCS / LTCS Galv / HDPE

Inputs needed: material_family

RAG sources: PIM-SU-3541-TCO

Line 59  ·  Color: BLU  ·  Pattern A+ B

Text: "Perform surface preparation and touch up field welds and any other coating damages in accordance with TES COM-SU-4743-TCO and coating system(s) per TES COM-SU-5191-TCO specified on drawings.."

Keep when: material_family = LTCS (only)

Remove when: Line 58c is kept or material_family = LTCS Galv / SS / HPDE

Inputs needed: coating_system, material_family, placeholders_dwg

RAG sources: COM-SU-4743-TCO, COM-SU-5191-TCO

Line 60  ·  Color: GRN ·  Pattern A + D

Text: " Apply Aerogel (Pyrogel) insulation on seam welds prior heat tracing installation in accordance with P-ST-6199."

Keep when: Line 45a and 51 are kept. (Note: both lines should be true)

Remove when: Otherwise

Inputs needed: heat_tracing, material_family, alloy_pmi_required

RAG sources: P-ST-6199."

Line 61a  ·  Color: GRN/PUR/RED  ·  Pattern A + C   (OR-split of Line 61)

Text: "Arrange with TTT I&E Coordinator [OR]  Zone Maintenance to reinstate / install related electric trace heating and instruments, as per I&E Job Pack YY-NNNN.1."

Keep when: Line 45a is kept.

Remove when: Heat tracing = None

Inputs needed: heat_tracing

Note: I&E Job Pack YY-NNNN.1 exact number should propagate from Line 45a and 45a rules apply here.

Line 61b  ·  Color: GRN/PUR/RED  ·  Pattern A + C   (OR-split of Line 61)

Text: "Reinstate hot water / steam tracing and conduct NDE as well as service testing of each separate loop.."

Keep when: Line 45b is kept

Remove when: Heat tracing = None

Inputs needed: heat_tracing

Note: OR-sibling of 61a; 45b rules apply here.

Line 62  ·  Color: BLU  ·  Pattern Always-include

Text: "All QM piping passport documentation (A-category Checklists) to be verified and signed by TCO-approved QM Inspector prior to PSSR, but not later than 24 hours after mechanical scope completion.

Keep when: Always

Remove when: Never

Note: Applicable if MOC is present, line will be removed potentially in rev3. Keep for now.

Line 63a  ·  Color: BLU/GRN/RED  ·  Pattern B + C

Text: "Advise xxxxx yyyyy/ zzzzz mmmm, DTEC KTL Plant Support / DTEC T/A / SGP/SGI, / FUPO Engineer at ext. 0000 about work completion and invite all necessary people to PSSR”

Keep when: Always

Remove when: Never

Note: If personalization of AI tool will be feasible this line will be perfect for implementation. For now keep it as it is.

Line 63b  ·  Color: BLU/GRN/RED  ·  Pattern B + C

Text: " NOTE: Arrange with RE Integrated Machinery Inspection (IMI) group (x3969/x7375) for inspection of suction/discharge flange alignment prior to torqueing of bolts to rotating equipment nozzles.”

Keep when: Line 44a is kept. (Almost never for LCW)

Remove when: Otherwise

Line 63b  ·  Color: BLU/GRN/RED  ·  Pattern B + C

Text: " NOTE: Arrange with RE Integrated Machinery Inspection (IMI) group (x3969/x7375) for inspection of suction/discharge flange alignment prior to torqueing of bolts to rotating equipment nozzles.”

Keep when: Line 18a is kept. (Almost never for LCW)

Remove when: Otherwise

Line 63c  ·  Color: BLU/GRN/RED  ·  Pattern B + C

Text: " NOTE: Fixed Equipment nozzle flange alignment to be witnessed and accepted stress-free by TCO Designs & Technical Engineering Center (DTEC) and RE QM Bolting Team prior to PSSR.”

Keep when: Line 18b is kept. (Almost never for LCW)

Remove when: Otherwise

Line 63d  ·  Color: BLU/GRN/RED  ·  Pattern B + C

Text: " NOTE: Machinery connection flange alignment to be witnessed and accepted stress-free by TCO Designs & Technical Engineering Center (DTEC) and RE QM Bolting Team prior to PSSR.”

Keep when: Line 18b is kept. (Almost never for LCW)

Remove when: Otherwise

Line 63d  ·  Color: BLU/GRN/RED  ·  Pattern B + C

Text: " NOTE: PSV connections flange alignment to be witnessed and accepted stress-free by TCO Designs & Technical Engineering Center (DTEC) and RE QM Bolting Team prior to PSSR.”

Keep when: Line 18c is kept. (Almost never for LCW)

Remove when: Otherwise

Line 63e  ·  Color: BLU/GRN/RED  ·  Pattern B + C

Text: " NOTE: Swing Elbow connection flange alignment to be witnessed and accepted stress-free by TCO Designs & Technical Engineering Center (DTEC) and RE QM Bolting Team prior to PSSR.”

Keep when: scope_type = swing elbow (Almost never for LCW)

Remove when: Otherwise

Line 64  ·  Color: BLU  ·  Pattern Always-include

Text: "Job Pack Coordinator shall submit the completed PIC check-list to Operations department after being signed by Quality Management group. (Form is provided by QM Dept.).."

Keep when: Line 62 is kept

Remove when: Otherwise

Line 65  ·  Color: BLU  ·  Pattern Always-include

Text: "Conduct PSSR and close out A-category punch points."

Keep when: Line 64 is kept

Remove when: Never

Line 66  ·  Color: GRN/BLU2/RED  ·  Pattern A + B + C

Text: "Following PSSR reinstate / install insulation and cladding, as per TES IRM-SU-1381-TCO and as specified on isometric drawing 00-YYYY-L-ZZZZ.”

Keep when: Line 42 is kept

Remove when: Otherwise

Inputs needed: insulation_type, placeholders_dwg

RAG sources: IRM-SU-1381.

Note: 00-YYYY-L-ZZZ number should propagate from Line 47a.

Note: ISO or Sketch ∈ INSULATION ≠ N/A. (Potentially, it is a result of phase 2 when drawing is parsed and tabular data retrieved.)

Line 67  ·  Color: GRN/BLU  ·  Pattern Always-include

Text: "Close out all B-category punch points and remove associated scaffolding."

Keep when: Line 66 is kept

Remove when: Otherwise

Line 68  ·  Color: BLU  ·  Pattern Always-include

Text: "Clean up the work area and demobilize from site."

Keep when: Always

Remove when: Never

## 5.10. AFTER COMPLETION OF WORK

Line 69  ·  Color: BLK  ·  Pattern Structural

Text: "AFTER COMPLETION OF WORK"

Keep when: Always

Remove when: Never

Line 70  ·  Color: GRN  ·  Pattern A

Text: "Perform service test."

Keep when: Line 34a is DROPPED.

Remove when: Line 34a is kept,

Inputs needed: service, hydrotest_method

Line 83  ·  Color: BLU  ·  Pattern Always-include

Text: "Final as-built package (including B-category Checklist) submitted to TCO QM group within 2 weeks."

Keep when: Always

Remove when: Never

Line 84  ·  Color: BLU  ·  Pattern Always-include

Text: "Return excess material to TCO Warehouse."

Keep when: Always

Remove when: Never

## 5.11. ATTACHMENTS

Line 85  ·  Color: BLU4/PUR  ·  Pattern Structural + D

Text: "ATTACHMENTS: [Generally in the order shown below; see sect 4 of READ ME]"

Keep when: Always (header). Strip bracket instruction.

Remove when: Never

## 5.12. REFERENCED TCO SPECIFICATIONS

Line 87-119  ·  Color: GRN  ·  Pattern A (topical-relevance, not strict citation-match)  ·  v1.1 UPDATED

Text: "List of 33 candidate TCO specifications."

Keep when: Specs that are EITHER (a) actively cited in body OR (b) governing references topically relevant to the work (e.g., A-ST-2008 always, PIM-SU-2505 for piping fab, SI-105 for permits)

Remove when: Specs not topically relevant to the scope

Inputs needed: citation_list_from_body, scope_type, material, service

RAG sources: AI body output + topical relevance lookup

Note: v1.1 CORRECTION: previous closed-loop strict rule was wrong. Real EWOs include governing references (A-ST-2008, SI-105, PIM-SU-2505, W-ST-2025, PIM-SU-5209) even when not verbatim-cited in body. Two-category logic: (1) actively-cited specs MUST appear in list; (2) governing-references SHOULD appear by topic. Cited-but-not-listed = ERROR. Listed-but-not-cited = ACCEPTABLE if topically governing.