PRESENT SITUATION:

Leak detected on 8-inch carbon steel line near exchanger E-101.

PROPOSED SOLUTION:

Replace the leaking 8-inch carbon steel piping near exchanger E-101 with in-kind material in accordance with line class 150H21, including required tie-ins at designated points, verification of routing and weld locations on site, and execution in compliance with TCO safety and quality standards.

SCOPE OF WORK

BUSINESS PARTNER (BP) shall strictly follow all relevant TCO Safety Instructions.

BUSINESS PARTNER (BP) shall follow all Quality Management (QM) requirements designated in the Quality Control package.

PRELIMINARY SITE WORK (PSW):

Prior to start of mechanical activities, perform wall thickness testing at TP01-123/000 and TP01-123/999.

NOTE: Perform the wall thickness test by marking out and recording a matrix as directed by the TCO FER Unit Inspector.

Prior to commencing any fabrication works verify dimensions, pipe routing and field weld locations on site.